# GlandBox

Wiring accessory for control and indicating equipment (CIE)

| ### Key features | Places to install cable glands 16 For routing cables that are not hidden inside the wall. There are four places on each side. Cable gland diameter up to 20 mm Cable glands are not included in the complete set. Recesses for drilling The drill bit is fixed in the recess and does not move during drilling. We recommend using a hole saw for plastic with a diameter of up to 20 mm. Sabotage protection tamper button The perforated part of GlandBox is used for the EN54 Fire Hub Jeweller tamper button to trigger when the CIE is installed in GlandBox. An Ajax system sends fault notifications if an attempt is made to detach GlandBox from the surface or remove the CIE from the mounting panel. Cable fasteners To hold the cables together with ties. Aids for easy installation - Bubble level - UP key - CIE center marking The bubble level allows for checking the casing inclination angle. The intersection of two lines marks the center of the CIE, and the UP key marks its top. |
| --- | --- |
| ### Recommendations | The accessory is designed for indoor use only. GlandBox is compatible only with EN54 Fire Hub Jeweller. |
| ### Product | Dimensions 263 × 223 × 40 mm 10.35" × 8.78" × 1.58" Weight 335 g 11.82 oz Operating temperature range from –10 °C to +40 °C from 14 °F to 104 °F Operating humidity up to 75% non-condensing Flammability rating UL 94 V-0 |
| ### Color | Black White |
| ### Complete set | GlandBox Installation kit Quick start guide |
| Warranty | 24 months |

Revision 1