# CurtainCam Outdoor HighMount (PhOD) Jeweller

Wireless dual-technology curtain motion detector with extended photo verification possibilities. For outdoor installation at a height of 2–4 m.

| ### Communication | Jeweller communication technology Proprietary wireless communication technology to transmit commands, alarms, and events. Key features: - Two-way communication. - Advanced sabotage protection. - Block encryption featuring a dynamic key. - Instant notifications. - Remote control and configuration in Ajax apps. Frequency bands 866.0–866.5 MHz 868.0–868.6 MHz 868.7–869.2 MHz 905.0–926.5 MHz 915.85–926.5 MHz 921.0–922.0 MHz Depends on the sales region. Maximum effective radiated power (ERP) up to 25 mW Automatic power control to reduce power consumption and radio interference. Radio signal modulation GFSK Radio communication range up to 1,700 m up to 5,550 ft Between the device and a hub (or a range extender) in open space. Encrypted communication All stored and transmitted data is protected by block encryption featuring a dynamic key. Frequency hopping To prevent radio interference and jamming. Wings communication technology Proprietary wireless communication technology to transmit large data packets. Key features: - Two-way communication. - Advanced sabotage protection. - Block encryption featuring a dynamic key. - Data packet reload in case of transmission errors. - Photo delivery check. Frequency bands 866.0–866.5 MHz 868.0–868.6 MHz 868.7–869.2 MHz 905.0–926.5 MHz 915.85–926.5 MHz 921.0–922.0 MHz Depends on the sales region. Radio communication range up to 1,700 m up to 5,550 ft Between the device and a hub (or a range extender) in open space. Encrypted communication All stored and transmitted data is protected by block encryption featuring a dynamic key. Frequency hopping To prevent radio interference and jamming. |
| --- | --- |
| ### Detection | Sensitive elements 2 × PIR sensor 1 × K-band microwave sensor Operating frequency: 24 GHz. Motion detection distance up to 15 m up to 49 ft When installed at a height of 2–4 m. When installed at a height of 6.6–13.1 ft. Depends on the sensitivity level adjusted in the device settings. Main motion detection zone angles horizontal: 7° vertical: 73° Pre-alarm motion detection zone angles horizontal: 43° vertical: 88° To catch an intruder on photos, the device can start taking photos when a threat is detected in the pre-alarm detection zone. However, the device does not raise an alarm if the intruder never crosses the main detection zone. Motion detection speed from 0.3 to 2.0 m/s from 1.0 to 6.6 ft/s The detector sensors should be directed perpendicular to the expected intrusion path into the facility. Sensitivity 3 levels Adjusted by a PRO or a user with admin rights in Ajax apps. False alarm prevention SmartDetect Software algorithm. Temperature compensation Effective motion detection at temperatures from −25 °C to +60 °C. Effective motion detection at temperatures from −13 °F to 140 °F. |
| ### Photo verification | Photo by alarm The built-in camera activates if the detector is armed and motion is detected. Photo on demand A user takes pictures at any time or when the detector is armed (depending on settings). The feature is disabled by default. Photo by scenario The detector takes pictures automatically when other Ajax detectors are triggered. The feature is not configured by default. Photo by schedule The detector automatically takes photos at the specified time. The feature is not configured by default. Photo by arming/disarming The detector automatically takes photos when the security state is changed. Adjustable photo resolution 640 × 480 px 320 × 240 px (by default) Adjusted by a PRO or a user with admin rights in Ajax apps. Camera viewing angles horizontal: 121° vertical: 92.5° HDR image Improves detail accuracy in bright and dark scenes. IR illumination To take photos in low-light conditions. Series of pictures from 1 to 5 images with a resolution of 320 × 240 px from 1 to 3 images with a resolution of 640 × 480 px Adjusted by a PRO or a user with admin rights in Ajax apps. Photo delivery time up to 9 s for 320 × 240 px resolution up to 20 s for 640 × 480 px resolution Photo delivery time depends on the image resolution selected in the device settings. |
| ### Recommendations | It is recommended to install the detector at a height of 2–4 m. It is recommended to install the detector at a height of 6.6–13.1 ft. The Pet immunity feature is not available for this device. The device is designed for outdoor and indoor use. |
| ### Sabotage protection | Tamper alarm Notifications of attempts to remove the detector from the mounting panel. Magnetometer detects attempts to tilt or shift the detector or detach it from the surface Anti-masking system detects attempts to block the detector view Holding screw secures the device to SmartBracket Protection against spoofing device authentication Communication loss detection after 36 s Detection time depends on the Jeweller or Jeweller/Fibra settings. |
| ### Main power supply | Battery 6 × CR123A Pre-installed. Type type C Calculated battery life up to 3 years Operating voltage range 3.8−6.5 V⎓ Nominal operating voltage 6 V⎓ Quiescent current consumption at nominal voltage 45 μA Maximum current consumption at nominal voltage 1,200 mA Full battery capacity 4,800 mAh Low battery voltage 4.2 V⎓ Low battery recovery voltage 5.0 V⎓ End-of-life battery voltage 3.8 V⎓ The device switches off. End-of-life battery capacity 300 mAh Manufacturer Huiderui |
| ### Optional power supply | External supply operating voltage 12−24 V⎓ The batteries serve as a backup when an external power supply is connected. |
| ### Enclosure | Dimensions with the swivel mount 234 × 98 × 275 mm 9.21" × 3.86" × 10.83" Dimensions without the swivel mount 234 × 98 × 137 mm 9.21" × 3.86" × 5.39" Weight with the swivel mount 1,109 g 2.44 lb Weight without the swivel mount 620 g 1.37 lb Operating temperature range from −25 °C to +60 °C from −13 °F to 140 °F Operating humidity up to 95% Protection class IP55 |
| ### Color | White |
| ### Complete set | CurtainCam Outdoor HighMount (PhOD) Jeweller 6 × CR123A battery (pre-installed) SmartBracket mounting panel Swivel mount Hood Installation kit Quick start guide |
| ### Warranty | 24 months |