# Curtain Outdoor Mini Jeweller

Wireless dual-technology curtain motion detector for outdoor and indoor use

| Communication | Jeweller communication technology Proprietary wireless communication technology to transmit commands, alarms, and events. Key features: - Two-way communication. - Advanced anti-sabotage. - Block encryption featuring a dynamic key. - Instant notifications. - Remote control and configuration in Ajax apps. Frequency bands 866.0–866.5 MHz 868.0–868.6 MHz 868.7–869.2 MHz 905.0–926.5 MHz 915.85–926.5 MHz 921.0–922.0 MHz Depends on the sales region. Maximum effective radiated power (ERP) up to 20 mW Automatic power control to reduce power consumption and radio interference. Radio signal modulation GFSK Radio communication range up to 1,700 m up to 5,550 ft Between the detector and a hub (or a range extender) in an open space. Encrypted communication All stored and transmitted data are protected by block encryption featuring a dynamic key. Frequency hopping To prevent radio interference and jamming. [icon] Wings communication technology Proprietary wireless communication technology for uploading firmware updates. Key features: - Two-way communication. - Advanced anti-sabotage. - Block encryption featuring a dynamic key. - Data packets reload in case of transmission errors. Frequency bands 866.0–866.5 MHz 868.0–868.6 MHz 868.7–869.2 MHz 905.0–926.5 MHz 915.85–926.5 MHz 921.0–922.0 MHz Depends on the sales region. Radio communication range up to 1,700 m up to 5,550 ft Between the detector and a hub (or a range extender) in an open space. Encrypted communication All stored and transmitted data are protected by block encryption featuring a dynamic key. Frequency hopping To prevent radio interference and jamming. |
| --- | --- |
| Detection | Sensitive elements 1 × PIR sensor 1 × K-band microwave sensor K-band microwave sensor to prevent false alarms caused by thermal interference. Operation frequency: 24 GHz. Motion detection distance up to 5 m up to 16.4 ft When installed at a height of 1 m. When installed at a height of 3.3 ft. Motion detection angles horizontal: 15° vertical: 100° Motion detection speed 0.3 to 2.0 m/s 1 to 6.6 ft/s The direction of the detector lens should be perpendicular to an alleged intrusion path into the facility. Sensitivity 3 levels Adjusted by a PRO or a user with admin rights in the Ajax apps. Pet immunity height: up to 100 cm (3.3 ft) weight: irrelevant The detector should be installed at a height of 1 meter, and the slider on its enclosure should be set to the Pet position. The detector should be installed at a height of 3.3 feet, and the slider on its enclosure should be set to the Pet position. False alarm prevention dual motion detection technology SmartDetect Software algorithm. Temperature compensation Effective motion detection at temperatures from −25 °C to +60 °C. (−13 °F to +140 °F) |
| Recommendations | The device's recommended installation height is 5 m on the ceiling and 1 m on the side. The device's recommended installation height is 16.4 ft on the ceiling and 3.3 ft on the side. The device is designed for indoor and outdoor use. |
| Sabotage protection | Tamper alarm Notifications on attempts to detach the detector from the surface or remove it from the mounting panel. Protection against spoofing device authentication Anti-masking system detects attempts to block the detector view Accelerometer alarm when the detector is moved Must be activated when the detector is installed on third-party brackets. Holding screw To secure the detector on SmartBracket. Communication loss detection after 36 s Detection time depends on Jeweller or Jeweller/Fibra settings. |
| Power supply | Battery 2 × CR123A Pre-installed. Type type C Calculated battery life up to 3 years Detector’s operating voltage range 4.1−6.2 V⎓ Detector’s nominal operating voltage 6 V⎓ Quiescent detector’s current consumption at nominal voltage 30 μA Maximum detector’s current consumption at nominal voltage 600 mA Full battery capacity 1,600 mAh Low battery voltage 4.48 V⎓ Low battery recovery voltage 5.14 V⎓ End-of-life battery voltage 4.1 V⎓ Device is switched off. End-of-life battery capacity 170 mAh Manufacturer Huiderui or Panasonic |
| Enclosure | Dimensions of detector 145 × 32 × 43 mm 5.71″ × 1.26″ × 1.69″ Dimensions of detector with angle mount 145 × 44 × 54 mm 5.71″ × 1.73″ × 2.13″ Weight of detector 130 g 4.59 oz Weight of angle mount 20 g 0.7 oz Operating temperature range from −25 °C to +60 °C from –13 °F to +140 °F Operating humidity up to 95% Protection class IP55 |
| Color | White |
| Complete set | Curtain Outdoor Mini Jeweller SmartBracket mounting panel 2 × CR123A battery (pre-installed) Angle mount Installation kit Quick start guide |