# Internal battery NB (7.2V/190Ah)

Pack of 20 non-rechargeable LTC ER34615 batteries to install in a compatible Ajax casing

| Battery type | Battery 20 × ER34615 Non-rechargeable lithium thionyl chloride battery with a HPC-1520 pulse capacitor. Nominal voltage 7.2 V⎓ Nominal capacity 190 Ah At a current consumption of 10 mA (with Battery power saver mode enabled), a temperature of 25 °C 77 °F, and correct placement of the battery in the Ajax casing. Low temperatures may reduce battery capacity and shorten the battery life. Maximum load current 700 mA End-of-life battery voltage 4 V⎓ |
| --- | --- |
| Cautions | Do not short the power terminals. Do not recharge the battery. Use the battery only with compatible devices. The battery is designed for indoor use only. Secure the battery in the casing using the hook-and-loop fastener (included in the Case D (430) and Case E (395) complete sets). |
| Product | Dimensions 182 × 124 × 64 mm 7.17″ × 4.88″ × 2.52″ Weight 2.1 kg 4.63 lb Battery connector type Molex 2139-2 or equivalent Operating temperature range from −25 °C to +65 °C from −13 °F to 149 °F Operating relative humidity from 40% to 80% |
| Complete set | Internal battery NB (7.2V/190Ah) |
| Warranty | 24 months |