# Internal battery RB (6.4V/78Ah)

Pack of 78 rechargeable LiFePO4 batteries to install in a compatible Ajax casing

| Battery type | Battery 78 × IFR18650 Rechargeable lithium iron phosphate (LiFePO4) battery cells. Battery configuration 2S39P Nominal voltage 6.4 V⎓ Nominal capacity 78 Ah Maximum charge current 15 A Via the battery charging connector. TVC (Terminal Voltage Control) 7.2 V⎓ (3.6 V⎓ per cell) Maximum discharge current 2 A Via the connector for attaching the battery to the hub. Discharge cut-off voltage 4.5 V⎓ |
| --- | --- |
| Charging and discharging parameters | Charging temperature range from 0 °C to +45 °C from 32 °F to 113 °F Charging relative humidity 60 ± 25% Discharging temperature range from −25 °C to +65 °C from −13 °F to 149 °F Discharging relative humidity 60 ± 25% |
| Storage conditions | Storage temperature range from −20 °C to +45 °C from −4 °F to 113 °F Storage humidity 60 ± 25% |
| Transportation and shipping | Shipping voltage 6.0–6.4 V⎓ Transportation requirements For air transportation, lithium batteries must be charged to no more than 30% of their rated capacity. The shipping voltage complies with this requirement. For other types of transportation, the battery must be handled in accordance with applicable regulations and carrier requirements. |
| Cautions | Do not expose the battery to open flames, sparks, or any other ignition source. Do not short the power terminals. Do not disassemble or deform the battery. Avoid excessive mechanical impact or vibration. Use the battery only with compatible devices. The hub does not charge this battery. Charge the battery using a dedicated external charger. Recharge the battery every six months during storage. The battery must be installed fully charged. Secure the battery in the casing using the hook-and-loop fastener (included in the Case D (430) and Case E (395) complete sets). |
| Product | Dimensions 254 × 134 × 52 mm 10.00″ × 5.27″ × 2.05″ Weight 3.51 kg 7.74 lb Battery charging connector XT60 or equivalent Connector for attaching the battery to the hub Molex 2139-2 or equivalent Operating temperature range from −25 °C to +65 °C from −13 °F to 149 °F Operating relative humidity 60 ± 25% |
| Complete set | Internal battery RB (6.4V/78Ah) |
| Warranty | 24 months |